Galileo Magnetometer, Ida Browse Data - Additional Documentation

================================================================

[These data were extracted from the separate catalog files originally

provided with the BROWSE data file.]

Dataset Overview

================

This dataset contains data acquired by the Galileo Magnetometer during the

Ida flyby on Aug 28, 1993. The browse dataset has been created by averaging

samples from the 4/3 sec optimal averager data to 20 second sampling and

merging the data with the 1 and 2 RIM averages of the optimal

averager to provide a continuous dataset for the encounter day. Limited space

on the tape recorder forced the magnetometer team to limit their highest time

resolution observations to an ~30 minute interval beginning after closest

approach to Ida. In order to acquire the high time resolution mag data at Ida

necessary to look for magnetic signatures similar to those observed at

Gaspra, a new method of using the instrument optimal averager section

was tested. In this method, the CDS sampled the MAG memory to retrieve data

every 4/3 second. The data were returned to the ground via a CDS memory

readout (MRO). The data were highly over-filtered by the instrument but many

of the effects of over-filtering were recovered in ground data processing by

using knowledge of the filter response function. The optimal averager section

of the instrument (please see the instrument description) was configured to

acquire 1 and 2 RIM (RIM = 60.667 sec) averages to cover the rest of the

encounter day. These data have been processed to remove most

instrument and filter response function characteristics from the data.

The magnetometer data are provided in heliographic (RTN) coordinates.

Trajectory data have been provided as a separate archive product.

Primary Dataset Reference:

Kivelson, M.G., Z. Wang, S. Joy, K.K. Khurana, C. Polanskey, D.J.

Southwood, and R.J. Walker, 'Solar Wind Interactions with Small

Bodies: 2. What Can Galileo's Detection of Magnetic Rotations Tell

Us About GASPRA and IDA', Advances in Space Research, 1994

Primary Instrument Reference:

Kivelson, M. G., K. K. Khurana, J. D. Means, C. T. Russell, and

R. C. Snare, 'The Galileo magnetic field investigation', Space Science

Reviews, 60, 1-4, 357, 1992.

Data Columns:

time    S/C event time (UT) given in PDS time format YYYY-MM-DDThh:mm:ss.sssZ

Br      Magnetic field radial component in RTN coordinates <nT>

Bt      Magnetic field tangential component in RTN coordinates <nT>

Bn      Magnetic field normal component in RTN coordinates <nT>

Bmag    Average magnetic field magnitude <nT>

Missing data value = 99999.999

Fortran Format of the data file: (1X, A24, 4F11.3)

Data Acquisition:

The data for this dataset were acquired by the outboard magnetometer (flip

left position) in the low field (+/- 32 nT) mode (ULLR). This mode

has a digitization step size of 0.0625 nanoTesla. However, these data are

acquired at 30 vectors/second and then recursively filtered in the

instrument. The high rate data that are recorded to tape have a sample rate of

4.5 vectors/second. If there is sufficient variation in the 6-7 input samples

that make up a single output sample, then the effective digitization step size

becomes much smaller. The data are next passed to an onboard processing

algorithm which corrects the data for offsets, gains, and geometry. The

data can now be sent to the tape recorder or passed to the optimal averager

section of the instrument.

Optimal averager data are decimated to 1 vector every minor frame such that

only the first sample of he minor frame is retained. These data are then

despun into Inertial Rotor Coordinates (IRC) and passed to another

recursive filter operation. The filter used in the Ida high resolution

(4/3 sec) data was:

Bo(i) = (15/16) * Bo(i-1) + (1/16)*Bi(i)

where Bo(i) is the output of the i-th sample and Bi(i) is the new input vector

at the time of the i-th sample. The 1 and 2 RIM optimal averager data were

more highly filtered (1/32 and 1/64 filters respectively).

Data Sampling:

The high rate optimal averager data have been resampled at 20 second

resolution where the time stamp indicates the center of the average. Averages

are non-overlapping.

The low rate optimal averager data are received on the ground with an end of

average time stamp. The timing of these data is then corrected to an earlier

time approximately 1/3 sampling interval earlier than the end of average. The

timing correction has been determined empirically from observations where

both high rate and optimal averager data were simultaneously acquired.

The time tag gives the spacecraft event time (SCET) in universal time (UT).

Coordinate System:

The data are provided in heliographic  RTN (radial-tangential-normal)

coordinates. The radial direction is taken along the instantaneous Sun->S/C

line, positive away from the sun. The tangential direction is found by taking

the cross product of the sun spin axis with the radial (T = Omega x R)

direction. Finally, the normal direction is the cross product of R and T

(N = R x T).

CONFIDENCE_LEVEL_NOTE

These data have been highly processed to obtain the final dataset. Both the

magnetometer data and the AACS data required to transform the mag data into

geophysical coordinates are of lower than average quality. The dataset should

not be used in wave studies. Much of the processing has been done in the

frequency domain and the spectra clearly show the effects. Overall, the field

values are probably correct to 0.01 nT and the AACS angles to better than

a degree. The segment of data that were recorded to tape (16:53 ->17:22) do

not carry as strong a warning.